HANDY SIZE BULK CARRIER 35,000 DWT-ITINERARY DATA

| Name of the vessel | M/V „Kotor“ IMO No. 9575981 |  |  |  |
| --- | --- | --- | --- | --- |
| Port of registry | Kotor Port of registry official number |  | 02-8/11 |  |
| Call sign | 4OCE | MMSI |  | 262005000 |
| Type of the vessel | Handy size bulk carrier 35k DWT, built in Shanghai Shipyard Co., Ltd –China |  |  |  |
| Owner | Crnogorska plovidba a.d. Kotor, Montenegro |  |  |  |
| Main engine | MAN B&W 6S42MC(MARK7);6,480 KW(8810ps)x136 RPM |  |  |  |
| Auxiliary Engines (3sets) | MAN B&W –CME Licence-5L 16/24; RPM 1200 r/m; rated output 500 KW; frequency 60 Hz |  |  |  |
| Flag | Montenegro |  |  |  |
| Class | BV / I HULL Bulk Carrier,CSR,BC (Holds Nos 2,4 may be empty), ESP,VeriSTAR-HULL,INWATERSURVEY,Grab(20) |  |  |  |
| DWT | s/dwt 35,000 mt on abt 10.8m Summer draft moulded |  |  |  |
| Light ship weight | 9600 t |  |  |  |
| Displacement summer | 44762,1 t |  |  |  |
| TPC (summer) | 46,5 t |  |  |  |
| Cargo holds | 5ha/5ho |  |  |  |
| LOA / bdth / depth | 179.90m / 28.40m / 15.00m |  |  |  |
| GRT / NRT | abt 22456 / abt 12060 |  |  |  |
| C.holds total capacity | Grain : abt 44439,1 m³ Bale : abt 42362,7 m³ |  |  |  |
| Cargo holds hatch covers operation: | electro-hydraulic folding type, each hatch cover is divided in 4 panels, each 2 panels shall be folded forward and aft. |  |  |  |
| Cranes | Electro-hydraulic 30.0mt x 4; sleewing angle:360 degrees endless; cranes located between the hatches; no grabs on board |  |  |  |
| Total fuel oil bunker capacity | F.O. abt 1098,1 t (85%) ; total diesel oil capacity : M.D.O.T abt 125 m³(85%) ; L.S.F.O.T abt 268,9 m³ (85%) m³ MGO abt 74,9 m³(85%) |  |  |  |
| Total ballast capacity | Abt 22668,9 m³(100%) |  |  |  |
| Service Speed | (d=10,00m,CSR(90%MCR) with 15% S.M.) : abt 13,7 knots |  |  |  |
| Laden sea voyage | ( for main engine only ), at normal rating (CSR) : abt 25,1 tons /day +5% |  |  |  |
| Ballast sea voyage | at 50% eco speed at 10.5 kts=abt13.9 t at 85% =abt 23,0 tons |  |  |  |
| Generators at sea | One generator running abt 2.4tons / day +5% |  |  |  |
| Port consumption | abt 5.50 t (with usage of cranes - two generators + boiler running) |  |  |  |
| In port when idle | abt 3.80 t ( one generator + boiler ) |  |  |  |
| CO² | Fixed CO² system for engine room and cargo holds protection |  |  |  |